| N830EH – Normal Checklist |  |  |
| --- | --- | --- |
| Engine Starting Preflight Inspection …...………. COMPLETE Passenger Briefing ……………..COMPLETE Seat Belts…………..….ADJUST AND LOCK Parking Brake ………………………..…… SET Electrical Equipment ……………………. OFF Master Switch ..………………………….. ON Nav Lights ………………..……………….. ON Propeller Area ……………………..… CLEAR Canopy .……………… LATCH AND LOCK Throttle…...…………………………. OPEN ¼” Fuel Valve ……………….……………….. ON Mixture ………………………………..…. RICH Starter ……………………………….ENGAGE Oil Pressure …..… CHECK (5 SECONDS) Throttle …..…………………..….. 1,200 RPM Avionics Master ……………………….... ON Flaps ………………..………………. RETRACT Mixture ………………………..………… LEAN Before Takeoff Parking Brake ……………..……………… SET Flight Controls …………………….... CHECK Instruments ........................... CHECK/SET GPS, Radio, Transponder …………….. SET Autopilot ………………….………………. TEST Mixture ………………..…….. AS REQUIRED Throttle ………………..…………. 2,000 RPM Magnetos ………….………………… CHECK Engine Instruments .……………….. CHECK Throttle …………..…………….. IDLE CHECK Throttle ……..……………………. 1,200 RPM Trim …………………..……………………… SET Canopy …………. LATCHED and LOCKED Normal Takeoff Canopy ………………...… VERIFY LOCKED Flaps …………………………..…………….. UP Lights ………………..……………………… ON Mixture …………………….... AS REQUIRED Throttle …………………………… FULL OPEN Elevator ………………..………….… LIFT TAIL Rotate ……………………………..…. 57 KIAS Initial Climb ………………...… XX KIAS (VY) Short Field Takeoff Canopy ……………...…… VERIFY LOCKED Flaps …………………………..…………….. XX Lights ……………..………………………… ON Mixture …………………….... AS REQUIRED Brakes ………………………………………. SET Throttle …………………………… FULL OPEN Brakes .………………………………. RELEASE Elevator ………. LIFT TAIL SLIGHTLY HIGH Rotate ……………………………..…. 52 KIAS Initial Climb ………………...… XX KIAS (VX) Airspeed Over Obstacle …. XX KIAS (VY) Flaps …………………………..…………….. UP | Climb Airspeed ……………………… XX – XX KIAS Lights …………………………. AS REQUIRED Mixture ……………………… AS REQUIRED Cruise Power …………………………………..…… SET Lights …………………………. AS REQUIRED Mixture ……………………… AS REQUIRED Fuel Quantity ………………..…. MONITOR Before Landing Fuel Valve …………………….... VERIFY ON Mixture …………………….... AS REQUIRED Seat Belts …………………….…... SECURED Lights …………………………..…………… ON Autopilot ………………………..………… OFF Normal Landing (3-Point) Traffic Pattern Airspeed …………. 80 KIAS Flaps ………………………….. AS REQUIRED Final Approach Airspeed …. 60–65 KIAS Throttle ……………….…..………….. CLOSED Touchdown …. FULL STALL - STICK BACK Brakes …………………..…… AS REQUIRED Normal Landing (Wheel Landing) Traffic Pattern Airspeed ……….… 80 KIAS Flaps ……..………………….. AS REQUIRED Final Approach Airspeed ….…... 60 KIAS Throttle ………..……….…………….. CLOSED Touchdown ..……… MAIN WHEELS FIRST Landing Roll .……. STICK BACK SLOWLY LOWER TAIL GENTLY Brakes …………………..…… AS REQUIRED Short Field Landing Traffic Pattern Airspeed …………. 70 KIAS Flaps ………………………..………………. 30° Final Approach Airspeed …. 52–57 KIAS Throttle ………..…………………….. CLOSED Touchdown …. FULL STALL - STICK BACK Brakes ……………..………… AS REQUIRED Balked Landing Throttle ……………………..……. FULL OPEN Climb Attitude …..… ESTABLISH POSITIVE Flaps ………..……………. RETRACT TO 10° Climb Airspeed ..…………….. XX–XX KIAS Flaps …..……………………………. RETRACT | After Landing Flaps ……………………..…………. RETRACT Lights ……..………………….. AS REQUIRED Pitot Heat …………….…………………... OFF Trim …………………..………………….. RESET Mixture ……………………… AS REQUIRED Shutdown Parking Brake ………………………..…… SET Avionics Master Switch ……………..… OFF Electrical Switches ……..………………. OFF Mixture ………………..…………….. CUTOFF Fuel Valve …..…………………………….. OFF Ignition Switch …………………..………. OFF Aircraft Times ……..…………………... NOTE Master Switch …….…………………….. OFF ELT ………………………….……. VERIFY OFF Control Stick …..…………………. SECURED Aircraft ………….…………………. SECURED (TIE DOWNS, CHOCKS, COVERS) [INSERT CRUISE POWER TABLE HERE] Airspeeds for Normal Operation Takeoff Rotation …………….. 52 - 57 KIAS Normal Climb ………………… XX -XX KIAS Best Rate-of-Climb (VY) ……….…. XX KIAS Best Angle-of-Climb (VX) ……….. XX KIAS Normal Landing (0°) .…………….. 65 KIAS Normal Landing (30°) ...………... 60 KIAS Short-Field Landing (30°) ..…….. 52 KIAS Maneuvering Speed (VA) .……. 108 KIAS Max Cruise Speed (VNO) ..…….. 118 KIAS Never Exceed Speed (VNE) .…... 171 KIAS Max Crosswind Component …… 15 KIAS N830EH–Aircraft Checklist ¦ Revision A |
| N830EH – Emergency Checklist |  |  |
| Engine Fire During Start Mixture ………………………………. CUTOFF Fuel Valve ………………..………………. OFF Throttle …………………..………. FORWARD Starter ………………………..………. CRANK Engine Failure Immediately After Takeoff Airspeed .….………………………… 70 KIAS Mixture ………………...……………. CUTOFF Fuel Valve ……………………………...…. OFF Ignition Switch …………………………... OFF Flaps …………………………. AS REQUIRED Master Switch …………………………… OFF Engine Failure In Flight Airspeed …………… BEST GLIDE, 70 KIAS Mixture ………………...……. AS REQUIRED Fuel Valve …………………..…………….. ON Ignition Switch …….… CHECK L, R, BOTH NOTE: If the propeller is windmilling, the engine will restart automatically within a few seconds. If the propeller has stopper, turn the ignition switch to START, advance throttle slowly from idle, and lean the mixture from full rich as required for smooth operation. Engine Fire In Flight Fuel Valve ……………………..………….. OFF Mixture …………………..…… IDLE CUTOFF Ignition Switch ……………..………….... OFF Master Switch ………………..…………. OFF Airspeed …………………………….. 70 KIAS If fire is not extinguished, increase airspeed Cabin Vents ……………………..….... CLOSE Landing Site …….…………………… SELECT Forced Landing Procedure ….... EXECUTE Emergency Landing Without Engine Power Emergency ……………..…………. DECLARE Fuel Valve ………………………..……….. OFF Mixture ……………..………… IDLE CUTOFF Ignition Switch ………..……………….... OFF Master Switch …..………………………. OFF Airspeed …………………………..… 60 KIAS Landing ………….... UPWIND, FULL STALL Brakes …….………………… APPLY HEAVILY ELT ………………..………………… ACTIVATE | Precautionary Landing With Engine Power Airspeed ………………….……..…… 74 KIAS Selected Field ………………….… FLY OVER AND INSPECT Emergency …………………..……. DECLARE Wing Flaps ……………….... AS REQUIRED Approach …………..………………. 60 KIAS Master Switch ………………………..…. OFF Landing ……………. UPWIND, FULL STALL Ignition Switch ……………………..….... OFF Brakes ……………………… APPLY HEAVILY ELT …………………………..……… ACTIVATE Ditching Emergency …………………..……. DECLARE Wing Flaps ……………………..………… 30° Power …………..………………. 300 Ft/Min AT 60 KIAS Approach .… Heavy Winds: INTO WIND Light Winds: PARALLEL TO SWELLS Touchdown ………………. LEVEL ATTITUDE Master Switch ……………………………. OFF ELT ………………………………..… ACTIVATE Airplane ……………………..….. EVACUATE Landing With a Flat Tire Approach………………………….. NORMAL Wing Flaps ……………………..………… 30° Touchdown …………... GOOD TIRE FIRST Hold airplane off flat tire with aileron Voltage/Amperage Issues Alternator Reset Button……..……….. PUSH Voltage Gauge ……………..……… CHECK If voltage and/or amperage still abnormal: Nonessential Equipment ……………… OFF Flight ……………………………. TERMINATE Airspeed/Altitude/ Attitude Unreliable Autopilot …..………………. DISCONNECT Pitot Heat ….……………………………… ON Autopilot Fault Autopilot …..………………. DISCONNECT Aircraft …………………..… FLY MANUALLY | Autopilot Jam Autopilot …..………………. DISCONNECT Flight Controls ……. APPLY BREAK-AWAY FORCE Aircraft ……………….……. FLY MANUALLY Autopilot …..……………….……… PULL C/B NOTE: When pilot applies force to flight control system, shear screw will break and safely disconnect flight controls from malfunctioning Autopilot servo. Once shear screw breaks, pilot needs to relax flight control input to avoid over correction. The flight controls will then operate normally. Spin Recovery Power ……………………………………… IDLE Ailerons ……..…………………….. NEUTRAL Rudder …………………..…… FULL RUDDER OPPOSITE ROTATION Elevator ………………… STICK FORWARD BRISKLY AFTER ¼ TURN Controls ………..……………… NEUTRALIZE Airspeeds for Emergency Operation Maneuvering Speed (VA) .…… 108 KIAS Best Glide Speed ..………………. 70 KIAS Emergency Landing ……………... 60 KIAS Bold faced type indicates immediate action items N830EH–Aircraft Checklist ¦ Revision A |