50-Hour Inspection Checklist

| Aircraft: N830EH Make: Krueger Evan M Model: Sonex B Serial Number: 0849B | Airframe TT: X.X Engine Tach: X.X Engine TT: X.X Prop TT: X.XX | Start Date: XX/XX/XXXX End Date: XX/XX/XXXX Performed By: Evan Krueger |
| --- | --- | --- |

Powerplant

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Inspect engine compartment (pre-cleaning) [JEM0002-12 Section 8.8] |  |  |  |
| Clean engine and engine compartment [JEM0002-12 Section 7.3] |  |  |  |
| Inspect starter and master solenoid connections [JEM0002-12 Section 8.8] |  |  |  |
| Inspect engine compartment including torque seal on through bolts [JEM0002-12 Section 8.8] |  |  |  |
| Inspect engine mounts and hardware for degradation and cracking [JEM0002-12 Section 8.8] |  |  |  |
| Inspect cylinders for oil leakage, cracks, discoloration, or damage [JEM0002-12 Section 8.8] |  |  |  |
| Inspect intake and exhaust system for damage and wear. [JEM0002-12 Section 8.8] |  |  |  |
| Inspect exhaust system springs and tabs [AC43.13 Section 8-49] |  |  |  |
| Inspect flywheel screws (no torque check) [JEM0002-12 Section 9.23] |  |  |  |
| Inspect crankcase and front seal for leaks [JEM0002-12 Section 8.8] |  |  |  |
| Inspect ignition coils, electrical connections, and coil gap [JEM0002-12 Section 3.4] |  |  |  |
| Inspect safety wires for condition and orientation [AC43.13 Section 7-124] |  |  |  |
| Inspect ram air ducts and baffles [JEM0002-12 Section 8.8] |  |  |  |
| Inspect engine controls and linkages for damage and smooth function [JEM0002-12 Section 8.8] |  |  |  |
| Inspect fuel lines, fittings, and fire sleeves for condition and leaks [AC43.13 Section 8-31] |  |  |  |
| Inspect fuel filter [AC43.13 Section 8-36] |  |  |  |
| Inspect air filter housing and air filter [ACV-R10-01 Manual] |  |  |  |
| Inspect AeroInjector for damage and rubber connector for cracking [ACV-C07 Manual] |  |  |  |
| Inspect oil hoses, lines, and fittings for condition and leaks [AC43.13 Section 8-5] |  |  |  |
| Check cylinder head bolt tension [JEM0002-12 Section 9.15] |  |  |  |
| Inspect top spring washer, valve springs, and rockers [JEM0002-12 Section 9.16] |  |  |  |
| Perform pre-inspection test run [JEM0002-12 Section 9.1] |  |  |  |
| Change oil and oil filter [JEM0002-12 Section 9.5] |  |  |  |
| Perform compression leak down check [JEM0002-12 Section 9.18] |  |  | #1: /80 ¦ #2: /80 ¦ #3: /80 ¦ #4: /80 ¦ #5: /80 ¦ #6: /80 |
| Check spark plug gaps [JEM0002-12 Section 3.4] |  |  |  |
| Measure fluid in oil separator and empty [ACV-S03-01 Manual] |  |  | Fluid Drained: |
| Conduct engine post inspection test run [JEM0002-12 Section 9.1] |  |  |  |

Propeller

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Inspect spinner for damage and security [JEM0002-12 Section 8.7] |  |  |  |
| Inspect propeller for damage, nicks, and corrosion [JEM0002-12 Section 8.7] |  |  |  |
| Check propeller bolts for proper torque [JEM0002-12 Section 8.7] |  |  |  |
| Perform propeller and spinner tracking procedures [JEM0002-12 Section 9.2] |  |  |  |

Airframe

| Inspection Item | Fail | Pass | Comments |
| --- | --- | --- | --- |
| Inspect welded components for cracks [SNX-SB-002] |  |  |  |

| Signature and Certificate # |  | Date |
| --- | --- | --- |

General Notes: